bitna razlika između “ima MQTT mogućnost” i “full MQTT broker platforme”.

Kratko: CrateDB nema MQTT broker. On može primati podatke koji dolaze iz MQTT-a, ali ne zamjenjuje broker poput EMQX-a.

Zašto CrateDB ≠ MQTT broker

CrateDB je:

- distributed SQL baza

- time-series + IoT storage

- real-time analytics engine

- geospatial DB

ali ne radi MQTT routing, session management ni device management

Što je MQTT broker (EMQX)

EMQX radi:

- MQTT connection handling (tisuće uređaja)

- session management (QoS 0/1/2)

- topic routing (trafostanice/TS-001)

- retained messages

- offline buffering

- authentication & ACL

- rule engine + integrations

- bridging prema DB-ovima

️ Ključna razlika

| Funkcija | EMQX (broker) | CrateDB (DB) |
| --- | --- | --- |
| MQTT konekcije | ✅ DA | ❌ NE |
| Device management | ✅ DA | ❌ NE |
| Routing poruka | ✅ DA | ❌ NE |
| Storage | ❌ NE | ✅ DA |
| SQL analitika | ❌ NE | ✅ DA |
| Geo upiti | ❌ NE | ✅ DA |
| AI/analytics | ❌ NE | ✅ DA |

Kako to stvarno radi u tvojoj arhitekturi

[Locust simulacija]
        ↓ MQTT
   ┌──────────────┐
   │   EMQX       │  ← MQTT broker (routing)
   └──────────────┘
        ↓ Rule Engine / Connector
   ┌──────────────┐
   │   CrateDB    │  ← storage + analytics
   └──────────────┘
        ↓
 FastAPI / AI / Vue dashboard

Bitna stvar

CrateDB može:

- primiti podatke preko SQL connectora

- integrirati se s EMQX Rule Engine

ALI:

ne može zamijeniti MQTT broker

Kada bi EMQX mogao “nestati”?

Možeš ga preskočiti samo ako:

Opcija A (jednostavni demo)

- Locust → HTTP → FastAPI → CrateDB

Opcija B (edge IoT minimal)

- direkt MQTT client → custom consumer → CrateDB

Ali za tvoj projekt (1000 trafostanica + SCADA)

EMQX ti daje:

- real MQTT backbone

- skalabilnost (10k–1M devices)

- topic routing po trafostanicama

- simulaciju realnog energetskog sustava

Najvažniji zaključak

CrateDB = mozak (storage + AI + analytics)
EMQX = živčani sustav (MQTT komunikacija)
Locust = generator svijeta (trafostanice)

može se napraviti CrateDB ↔ EMQX integracija, ali bitno je razumjeti jednu stvar:

ne postoji “native CrateDB connector” u EMQX-u kao službeni plug-and-play driver
 ali EMQX je dizajniran da se lako povezuje s bazama kroz Rules Engine + Actions

Dakle: da, radi se preko EMQX Data Integration layera.

✔ Najbolji način (standard industrijski)

Koristiš:

- EMQX → Rule Engine

- CrateDB → SQL storage (PostgreSQL protocol)

Kako EMQX → CrateDB zapravo radi

CrateDB ima PostgreSQL wire protocol, pa EMQX ne treba “poseban driver”.

koristi se PostgreSQL connector u EMQX-u

️ Arhitektura

MQTT Devices (Locust)
        ↓
   EMQX Broker
        ↓
 Rule Engine (SQL transform)
        ↓
PostgreSQL Connector
        ↓
     CrateDB

️ Koraci setup-a

1. CrateDB endpoint

CrateDB sluša kao PostgreSQL:

host: cratedb
port: 5432
user: crate
database: doc

2. EMQX Rule (SQL filter)

U EMQX Dashboard:

SELECT
  payload.timestamp AS ts,
  payload.station_id AS station_id,
  payload.station_name AS station_name,
  payload.location AS location,
  payload.electrical AS electrical,
  payload.thermal AS thermal,
  payload.oil_gas AS oil_gas,
  payload.alarms AS alarms
FROM
  "trafostanice/+/sensors"

ovo je “stream transform layer”

3. EMQX Action → PostgreSQL (CrateDB)

U EMQX:

Connector type:

- PostgreSQL

Config:

Host: cratedb
Port: 5432
Username: crate
Password: (empty)
Database: doc

4. SQL INSERT action

INSERT INTO trafostanice_sensors (
  timestamp,
  station_id,
  station_name,
  location,
  electrical,
  thermal,
  oil_gas,
  alarms
)
VALUES (
  ${ts},
  ${station_id},
  ${station_name},
  ${location},
  ${electrical},
  ${thermal},
  ${oil_gas},
  ${alarms}
)

Bitna stvar (CrateDB JSON mapping)

CrateDB voli OBJECT strukture, ali EMQX šalje JSON.

zato moraš osigurati da:

- location → GEO_POINT [lon, lat]

- objekti ostanu JSON-compatible

Alternativa (bolja u praksi)

Ako EMQX SQL mapping postane limit:

opcija B — HTTP bridge (češće u realnim sustavima)

EMQX Rule → HTTP POST → FastAPI → CrateDB

fleksibilnije
 lakše debugiranje
možeš dodati AI preprocessing

Najbolja praksa (realni SCADA / IoT)

Opcija 1 (clean & simple)

EMQX → PostgreSQL connector → CrateDB

Opcija 2 (enterprise)

EMQX → HTTP API → FastAPI → CrateDB + AI

Opcija 3 (heavy industry)

EMQX → Kafka → stream processing → CrateDB

Moj savjet za tvoj projekt

Pošto već imaš:

- 1000 trafostanica

- AI anomaly detection

- forecasting

- Vue SCADA dashboard

najbolja opcija je:

EMQX → FastAPI → CrateDB

Zašto:

- možeš ubaciti AI scoring prije spremanja

- lakše radiš predictive maintenance

- fleksibilniji schema evolution

- lakše debugiraš streaming podatke

Zaključak

- Da, EMQX ↔ CrateDB connector je moguć

- Radi se preko PostgreSQL integration ili HTTP bridge-a

- Nema “official CrateDB plugin”, ali nije ni potreban