Cilj sustava

Napraviti IoT/SCADA demo platformu koja:

- simulira trafostanice u Čakovcu

- generira MQTT podatke senzora

- sprema podatke u CrateDB

- koristi EMQX kao MQTT broker

- koristi Locust za simulaciju senzora

- ima realtime dashboard u Vue.js

- prikazuje geo lokacije trafostanica na mapi

- prikazuje električne senzore i alarme

Tehnologije

Backend / IoT

| Komponenta | Tehnologija |
| --- | --- |
| MQTT broker | EMQX |
| Simulator | Locust |
| Database | CrateDB |
| API backend | FastAPI |
| Realtime | WebSocket |

Frontend

| Komponenta | Tehnologija |
| --- | --- |
| Frontend | Vue 3 |
| Build | Vite |
| Mapa | Leaflet |
| State | Pinia |
| Grafovi | ECharts |

Što će simulirati trafostanice

Električni senzori

- napon

- struja

- frekvencija

- aktivna snaga

- jalova snaga

- harmonici

Temperatura

- temperatura ulja

- temperatura namota

- temperatura sabirnica

- ambijentalna temperatura

Ulje i plinovi

- razina ulja

- tlak ulja

- vlaga

- vodik

- metan

- acetilen

Dummy trafostanice Čakovec

| ID | Naziv | Lokacija |
| --- | --- | --- |
| TS-001 | TS Centar | Čakovec centar |
| TS-002 | TS Jug | Jug |
| TS-003 | TS Sjever | Sjever |
| TS-004 | TS Istok | Istok |
| TS-005 | TS Zapad | Zapad |

Arhitektura

Projekt struktura

Prvi korak

Prvo dižemo infrastrukturu:

- CrateDB

- EMQX

- Backend

- Vue frontend

- Locust simulator

Docker compose

Početni docker-compose.yml

| version: '3.9' services: cratedb: image: crate:latest container_name: cratedb ports: - "4200:4200" - "5432:5432" command: > crate -Ccluster.name=trafostanice -Cnode.name=node1 -Cauth.host_based.enabled=false emqx: image: emqx/emqx:latest container_name: emqx ports: - "1883:1883" - "18083:18083" backend: build: ./backend container_name: backend ports: - "8000:8000" depends_on: - cratedb - emqx frontend: build: ./frontend container_name: frontend ports: - "5173:5173" depends_on: - backend |  |
| --- | --- |

CrateDB tablica

cratedb/init.sql

| CREATE TABLE trafostanice_sensors ( timestamp TIMESTAMP, station_id TEXT, station_name TEXT, location GEO_POINT, electrical OBJECT(DYNAMIC) AS ( voltage_kv DOUBLE, current_a DOUBLE, frequency_hz DOUBLE, active_power_kw DOUBLE, reactive_power_kvar DOUBLE, harmonics_thd DOUBLE ), thermal OBJECT(DYNAMIC) AS ( oil_temp_c DOUBLE, winding_temp_c DOUBLE, busbar_temp_c DOUBLE, ambient_temp_c DOUBLE ), oil_gas OBJECT(DYNAMIC) AS ( oil_level_percent DOUBLE, oil_pressure_bar DOUBLE, humidity_ppm DOUBLE, hydrogen_ppm DOUBLE, methane_ppm DOUBLE, acetylene_ppm DOUBLE ) ); |  |
| --- | --- |

MQTT payload

Primjer poruke:

| { "timestamp": "2026-05-16T12:00:00Z", "station_id": "TS-001", "station_name": "TS Centar", "location": [16.4339, 46.3844], "electrical": { "voltage_kv": 10.2, "current_a": 240, "frequency_hz": 50.01, "active_power_kw": 1800, "reactive_power_kvar": 210, "harmonics_thd": 2.1 }, "thermal": { "oil_temp_c": 64, "winding_temp_c": 72, "busbar_temp_c": 43, "ambient_temp_c": 20 }, "oil_gas": { "oil_level_percent": 88, "oil_pressure_bar": 1.3, "humidity_ppm": 18, "hydrogen_ppm": 10, "methane_ppm": 4, "acetylene_ppm": 0 } } |  |
| --- | --- |

Dummy trafostanice — Čakovec

Primjer dummy geo lokacija

| [ { "id": "TS-001", "name": "TS Jug", "lat": 46.3812, "lon": 16.4321 }, { "id": "TS-002", "name": "TS Centar", "lat": 46.3851, "lon": 16.4358 }, { "id": "TS-003", "name": "TS Sjever", "lat": 46.3925, "lon": 16.4290 }, { "id": "TS-004", "name": "TS Istok", "lat": 46.3878, "lon": 16.4450 }, { "id": "TS-005", "name": "TS Zapad", "lat": 46.3840, "lon": 16.4200 } ] |  |
| --- | --- |

Senzori koje simuliraš

Električni senzori

| { "voltage_kv": 10.2, "current_a": 245, "frequency_hz": 49.98, "active_power_kw": 1850, "reactive_power_kvar": 220, "harmonics_thd": 2.1 } |  |
| --- | --- |

Temperatura

| { "oil_temp_c": 63, "winding_temp_c": 71, "busbar_temp_c": 45, "ambient_temp_c": 18 } |  |
| --- | --- |

Ulje i plinovi

| { "oil_level_percent": 87, "oil_pressure_bar": 1.2, "humidity_ppm": 22, "hydrogen_ppm": 15, "methane_ppm": 4, "acetylene_ppm": 0 } |  |
| --- | --- |

MQTT topic struktura

trafostanice/TS-001/sensors
trafostanice/TS-002/sensors
trafostanice/TS-003/sensors

Primjer MQTT poruke

| { "timestamp": "2026-05-16T12:00:00Z", "station_id": "TS-001", "geo": { "lat": 46.3812, "lon": 16.4321 }, "electrical": { "voltage_kv": 10.1, "current_a": 233, "frequency_hz": 50.01, "active_power_kw": 1720 }, "thermal": { "oil_temp_c": 66, "winding_temp_c": 74 }, "oil_gas": { "oil_level_percent": 91, "hydrogen_ppm": 12 } } |  |
| --- | --- |

Locust simulator primjer

| from locust import User, task, between import json import random import time import paho.mqtt.publish as publish stations = [ { "id": "TS-001", "lat": 46.3812, "lon": 16.4321 }, { "id": "TS-002", "lat": 46.3851, "lon": 16.4358 } ] class TrafostanicaUser(User): wait_time = between(1, 3) @task def send_sensor_data(self): station = random.choice(stations) payload = { "timestamp": time.time(), "station_id": station["id"], "geo": { "lat": station["lat"], "lon": station["lon"] }, "electrical": { "voltage_kv": round(random.uniform(9.8, 10.5), 2), "current_a": random.randint(150, 400), "frequency_hz": round(random.uniform(49.8, 50.2), 2), "active_power_kw": random.randint(1000, 3000), "reactive_power_kvar": random.randint(100, 500), "harmonics_thd": round(random.uniform(1.0, 5.0), 2) }, "thermal": { "oil_temp_c": random.randint(45, 85), "winding_temp_c": random.randint(50, 95), "ambient_temp_c": random.randint(-5, 35) }, "oil_gas": { "oil_level_percent": random.randint(70, 100), "oil_pressure_bar": round(random.uniform(1.0, 2.0), 2), "humidity_ppm": random.randint(5, 40), "hydrogen_ppm": random.randint(0, 25), "methane_ppm": random.randint(0, 10), "acetylene_ppm": random.randint(0, 3) } } publish.single( f"trafostanice/{station['id']}/sensors", payload=json.dumps(payload), hostname="localhost" ) |  |
| --- | --- |

Što dalje

Sljedeći koraci su:

1. Locust simulator

- generiranje MQTT podataka

2. EMQX Rule Engine

- MQTT → CrateDB

3. FastAPI backend

- REST API

- WebSocket

4. Vue dashboard

- Leaflet mapa

- live senzori

- alarmi

- grafovi

Konačni rezultat

Dobit ćeš:

- live SCADA-like sustav

- realtime mapa Čakovca

- simulaciju trafostanica

- MQTT streaming

- CrateDB storage

- Vue dashboard

- alarm monitoring

- geolokacije trafostanica

- mogućnost skaliranja na tisuće uređaja

窗体顶端

窗体底端