Da bi u Locustu simulirao alarme i kvarove, ne radiš samo random telemetry — nego uvodiš:

- abnormalna stanja

- fault injection

- event escalation

- blackout scenarije

- degradaciju kroz vrijeme

To je zapravo “digital twin event simulation”.

- Koncept

Normalno stanje:

Alarm stanje:

- Vrste kvarova koje simuliraš

| Fault Type | Primjer |
| --- | --- |
| Overload | previsoka struja |
| Overheating | oil temp > 95°C |
| Voltage drop | pad napona |
| Harmonics spike | THD abnormal |
| Gas anomaly | hydrogen spike |
| Cooling failure | rapid temp growth |
| Transformer degradation | lifetime decay |
| Blackout cascade | overload propagation |

- Primjer fault injection logike

Ideja

90% vremena:

- normal telemetry

10% vremena:

- fault scenario

# Primjer Locust fault simulatora

| import random import time from locust import User, task class TransformerSimulator(User): @task def send_telemetry(self): # normal values voltage = random.uniform(108, 112) current = random.uniform(300, 500) oil_temp = random.uniform(55, 75) hydrogen = random.uniform(20, 80) alarms = [] # RANDOM FAULT INJECTION fault = random.random() # OVERLOAD if fault < 0.03: current = random.uniform(700, 1000) alarms.append("OVERLOAD") # OVERHEATING elif fault < 0.06: oil_temp = random.uniform(95, 120) alarms.append("OVERHEAT_CRITICAL") # GAS ANOMALY elif fault < 0.08: hydrogen = random.uniform(400, 900) alarms.append("GAS_ANOMALY") # VOLTAGE INSTABILITY elif fault < 0.10: voltage = random.uniform(85, 95) alarms.append("VOLTAGE_DROP") payload = { "timestamp": time.time(), "station_id": "TS-001", "electrical": { "voltage_kv": voltage, "current_a": current }, "thermal": { "oil_temp_c": oil_temp }, "oil_gas": { "hydrogen_ppm": hydrogen }, "alarms": alarms } print(payload) |  |
| --- | --- |

- Simulacija degradacije (realnije)

Umjesto random faulta:

kvar raste kroz vrijeme.

Primjer

self.degradation += 0.1

Kad degradation prijeđe threshold:

if self.degradation > 80:
    oil_temp += 25
    hydrogen += 300

- Blackout propagation simulation

Jedna trafostanica padne

Primjer

if station_fault:

    neighbor.current *= 1.25

    if neighbor.current > 900:
        neighbor.alarm = "CASCADE_OVERLOAD"

- AI anomaly simulation

Možeš generirati:

- subtle anomalies

- noise

- drifting sensors

Primjer drift senzora

oil_temp += self.age * 0.02

- DGA fault simulation (vrlo realno)

Partial discharge

{
  "hydrogen": 650,
  "acetylene": 120
}

Thermal fault

{
  "methane": 400
}

- Alarm severity simulation

if oil_temp > 90:
    severity = "WARNING"

if oil_temp > 105:
    severity = "CRITICAL"

if oil_temp > 120:
    severity = "EMERGENCY"

- Dynamic scenarios

Heatwave

ambient_temp += 15

Peak consumption

current *= 1.4

Cooling fan failure

cooling_efficiency = 0.4

- Realistic utility behavior

Dan/noć load curve

Noću:

Dan:

Peak:

- Realtime flow

- Rezultat

Dobivaš:

- realistične kvarove

- cascading failures

- AI anomaly training data

- predictive maintenance dataset

- SCADA alarm scenarios

- Digital Twin stress testing