Locust simulator za 1000 trafostanica

Napravit ćemo simulator koji:

- generira 1000 dummy trafostanica oko Čakovca

- šalje MQTT podatke u EMQX

- simulira električne senzore

- simulira temperaturu i ulje/plinove

- generira realistične vrijednosti

- može simulirati alarme i kvarove

Koristit ćemo:

- Locust

- Paho MQTT

Struktura

requirements.txt

locust
paho-mqtt
faker

stations.py

Ovaj file generira 1000 trafostanica oko Čakovca.

| import random stations = [] CENTER_LAT = 46.3844 CENTER_LON = 16.4339 for i in range(1000): lat_offset = random.uniform(-0.05, 0.05) lon_offset = random.uniform(-0.05, 0.05) station = { "id": f"TS-{i+1:04}", "name": f"Trafostanica {i+1}", "lat": round(CENTER_LAT + lat_offset, 6), "lon": round(CENTER_LON + lon_offset, 6) } stations.append(station) |  |
| --- | --- |

locustfile.py

| from locust import User, task, between import random import json import time import paho.mqtt.publish as publish from stations import stations class TrafostanicaUser(User): wait_time = between(0.1, 1) @task def send_sensor_data(self): ts = random.choice(stations) overload = random.random() < 0.02 overheating = random.random() < 0.01 voltage = round(random.uniform(9.8, 10.5), 2) current = random.randint(120, 400) oil_temp = random.randint(45, 80) if overload: current = random.randint(450, 700) if overheating: oil_temp = random.randint(90, 120) payload = { "timestamp": time.strftime( "%Y-%m-%dT%H:%M:%SZ", time.gmtime() ), "station_id": ts["id"], "station_name": ts["name"], "location": [ ts["lon"], ts["lat"] ], "electrical": { "voltage_kv": voltage, "current_a": current, "frequency_hz": round( random.uniform(49.8, 50.2), 2 ), "active_power_kw": random.randint( 1000, 5000 ), "reactive_power_kvar": random.randint( 100, 800 ), "harmonics_thd": round( random.uniform(1, 5), 2 ) }, "thermal": { "oil_temp_c": oil_temp, "winding_temp_c": oil_temp + random.randint(5, 20), "busbar_temp_c": random.randint(30, 70), "ambient_temp_c": random.randint(-10, 35) }, "oil_gas": { "oil_level_percent": random.randint(70, 100), "oil_pressure_bar": round( random.uniform(1.0, 2.0), 2 ), "humidity_ppm": random.randint(5, 50), "hydrogen_ppm": random.randint(0, 20), "methane_ppm": random.randint(0, 10), "acetylene_ppm": random.randint(0, 3) }, "alarms": { "overload": overload, "overheating": overheating } } publish.single( topic=f"trafostanice/{ts['id']}/sensors", payload=json.dumps(payload), hostname="emqx", port=1883 ) |  |
| --- | --- |

Pokretanje

Instalacija

pip install -r requirements.txt

Start Locusta

locust -f locustfile.py

Web UI

http://localhost:8089

Preporučene postavke

Number of users

100

Spawn rate

10

Što dobiješ

Simulator će:

- simulirati 1000 trafostanica

- slati MQTT poruke

- generirati:

  - normalna stanja

  - overload

  - overheating

  - alarmne događaje

MQTT topicovi

trafostanice/TS-0001/sensors
trafostanice/TS-0002/sensors
...
trafostanice/TS-1000/sensors

Primjer poruke

| { "station_id": "TS-0042", "station_name": "Trafostanica 42", "location": [16.4421, 46.3822], "electrical": { "voltage_kv": 10.2, "current_a": 312, "frequency_hz": 50.01, "active_power_kw": 3200, "reactive_power_kvar": 220, "harmonics_thd": 2.1 }, "thermal": { "oil_temp_c": 67, "winding_temp_c": 82 }, "alarms": { "overload": false, "overheating": false } } |  |
| --- | --- |

Realistične simulacije

Simulator već podržava:

- random opterećenje

- temperaturne promjene

- harmonike

- DGA plinove

- alarmne događaje

Sljedeći korak

Sada ide:

2. EMQX → CrateDB integracija

Napravit ćemo:

- MQTT ingestion

- Rule Engine

- automatski insert u CrateDB

- realtime storage

- retention

- indexing

- geo queries.