Korak 5 — Realtime Analytics + AI Anomaly Detection + Predictive Maintenance

Sada već imaš pravi IoT/SCADA stack:

- CrateDB

- EMQX

- Locust

- Vue.js

- Quasar Framework

Sada dodajemo:

- realtime analytics

- anomaly detection

- predictive maintenance

- health score trafostanice

- AI alarm scoring

Ovo već postaje pravi Smart Grid / Digital Twin sustav.

Što ćemo analizirati

Iz podataka koje već imamo:

Elektrika

- napon

- struja

- harmonici

- snaga

Termalno

- temperatura ulja

- temperatura namota

Ulje/plinovi

- vodik

- metan

- acetilen

- vlaga

Cilj AI layera

Detektirati:

- overheating

- overload

- degradaciju izolacije

- lošu kvalitetu energije

- rizik kvara transformatora

- abnormalne trendove

Nova arhitektura

1. CrateDB realtime analytics

Average load

| SELECT station_id, AVG(electrical['current_a']) AS avg_current, AVG(thermal['oil_temp_c']) AS avg_temp, AVG(electrical['harmonics_thd']) AS avg_thd FROM trafostanice_sensors GROUP BY station_id; |  |
| --- | --- |

2. Health score

Računamo health score:

- 100 = zdravo

- 0 = kritično

SQL health formula

| SELECT station_id, 100 - AVG(thermal['oil_temp_c']) * 0.4 - AVG(electrical['harmonics_thd']) * 5 - AVG(oil_gas['hydrogen_ppm']) * 0.8 AS health_score FROM trafostanice_sensors GROUP BY station_id ORDER BY health_score ASC; |  |
| --- | --- |

3. Anomaly detection

Detektiramo:

- previsoku temperaturu

- abnormalne harmonike

- nagli skok struje

- DGA abnormalnosti

SQL anomaly query

| SELECT * FROM trafostanice_sensors WHERE thermal['oil_temp_c'] > 90 OR electrical['harmonics_thd'] > 4.5 OR oil_gas['hydrogen_ppm'] > 15 OR electrical['current_a'] > 600; |  |
| --- | --- |

4. Predictive maintenance

Ideja:

- gledamo trend temperature

- trend plinova

- trend opterećenja

- degradaciju kroz vrijeme

Failure risk score

| SELECT station_id, AVG(thermal['oil_temp_c']) * 0.35 + AVG(oil_gas['hydrogen_ppm']) * 1.5 + AVG(electrical['harmonics_thd']) * 4 + AVG(electrical['current_a']) * 0.05 AS failure_risk FROM trafostanice_sensors GROUP BY station_id ORDER BY failure_risk DESC; |  |
| --- | --- |

5. AI analytics service

Dodaj novi servis:

requirements.txt
pandas
numpy
scikit-learn
asyncpg
fastapi
uvicorn

anomaly.py

Koristimo Isolation Forest.

| from sklearn.ensemble import IsolationForest import pandas as pd def detect_anomalies(df): features = [ 'current_a', 'oil_temp_c', 'harmonics_thd', 'hydrogen_ppm' ] model = IsolationForest( contamination=0.02, random_state=42 ) df['anomaly'] = model.fit_predict( df[features] ) return df |  |
| --- | --- |

predictive.py
Simple predictive maintenance score.

| def calculate_health(row): score = 100 score -= row['oil_temp_c'] * 0.4 score -= row['harmonics_thd'] * 5 score -= row['hydrogen_ppm'] * 1.2 score -= row['current_a'] * 0.03 return max(score, 0) |  |
| --- | --- |

6. Analytics API

main.py

| from fastapi import FastAPI app = FastAPI() @app.get("/analytics/health") async def health_scores(): return { "critical": 23, "warning": 88, "healthy": 889 } @app.get("/analytics/anomalies") async def anomalies(): return [ { "station_id": "TS-0442", "risk": "critical", "reason": "high oil temp" } ] |  |
| --- | --- |

7. Dashboard Analytics Panel

Dodaj:

- health score

- risk ranking

- anomaly alerts

- transformer ranking

Quasar Analytics Card

| <template> <q-card dark class="bg-orange" > <q-card-section> <div class="text-h6"> AI Analytics </div> <div> Critical: {{ analytics.critical }} </div> <div> Warning: {{ analytics.warning }} </div> <div> Healthy: {{ analytics.healthy }} </div> </q-card-section> </q-card> </template> |  |
| --- | --- |

8. Smart SCADA Features

Sada možeš:

- rangirati trafostanice

- predvidjeti kvarove

- otkriti abnormalnosti

- pratiti health score

- vidjeti toplinsku degradaciju

- detektirati lošu izolaciju

9. Realtime Heatmap

CrateDB GEO_POINT + Leaflet:

- heatmap temperature

- overload zones

- harmonics density

- outage clustering

10. Napredna AI ideja

Kasnije možeš dodati:

- LSTM forecasting

- Transformer models

- Graph Neural Networks

- Digital Twin

- Grid balancing AI

- Power demand prediction

Što sada imaš

Praktički:

- IoT platformu

- Smart Grid simulator

- SCADA dashboard

- realtime analytics

- AI anomaly detection

- predictive maintenance

- geo analytics

- realtime streaming

- distributed TSDB

Sljedeći korak

Možemo sada dodati:

7. AI forecasting

- load prediction

- blackout prediction

- transformer lifetime prediction

- 1 Arhitektura AI forecasting sloja
ad dodajemo forecasting sloj koji radi na povijesnim podacima iz CrateDB i daje predikcije:

- load prediction (opterećenje)

- blackout prediction (rizik nestanka)

- transformer lifetime prediction (preostali životni vijek)

Koristit ćemo:

- CrateDB (data source + agregacije)

- scikit-learn (baseline ML)

- (kasnije možeš dodati PyTorch / LSTM ako želiš upgrade)

7. 2 Feature set (što model gleda)

Iz tvoje simulacije (Locust + EMQX):

Input features

Elektrika

- current_a

- voltage_kv

- active_power_kw

- harmonics_thd

Termalno

- oil_temp_c

- winding_temp_c

Ulje / plinovi

- hydrogen_ppm

- methane_ppm

- acetylene_ppm

Derived features

- load_factor

- temp_gradient

- gas_growth_rate (trend)

volatility (std dev)


7.3 Feature engineering (Python)

| import pandas as pd import numpy as np def build_features(df): df = df.sort_values("timestamp") df["load_factor"] = df["current_a"] / 500 # nominal df["temp_diff"] = df["oil_temp_c"] - df["ambient_temp_c"] df["gas_index"] = ( df["hydrogen_ppm"] + df["methane_ppm"] * 2 + df["acetylene_ppm"] * 5 ) df["rolling_temp"] = df["oil_temp_c"].rolling(10).mean() df["rolling_load"] = df["current_a"].rolling(10).mean() return df.dropna() |  |
| --- | --- |

7.4 LOAD PREDICTION (opterećenje)

Model ideja

- predviđamo budući current_a (15–60 min ahead)

Model (baseline)

| from sklearn.ensemble import RandomForestRegressor features = [ "voltage_kv", "active_power_kw", "harmonics_thd", "rolling_load" ] target = "current_a" model = RandomForestRegressor( n_estimators=200, random_state=42 ) |  |
| --- | --- |

Train

| def train_load_model(df): X = df[features] y = df[target] model.fit(X, y) return model |  |
| --- | --- |

Prediction

| def predict_load(model, latest_row): return model.predict([latest_row])[0] |  |
| --- | --- |

# 8. BLACKOUT PREDICTION (RIZIK ISPADA)

Ovo je classification problem:

- 0 = normal

- 1 = blackout risk

Label definition (heuristika)

| def create_blackout_label(df): df["blackout_risk"] = ( (df["voltage_kv"] < 9.5) ¦ (df["current_a"] > 650) ¦ (df["oil_temp_c"] > 95) ¦ (df["harmonics_thd"] > 5) ).astype(int) return df |  |
| --- | --- |

Model

from sklearn.ensemble import GradientBoostingClassifier

model = GradientBoostingClassifier()

Train

| features = [ "current_a", "voltage_kv", "oil_temp_c", "harmonics_thd", "gas_index" ] def train_blackout_model(df): X = df[features] y = df["blackout_risk"] model.fit(X, y) return model |  |
| --- | --- |

Output
{
  "station_id": "TS-042",
  "blackout_probability": 0.87,
  "risk_level": "CRITICAL"
}

# 9.TRANSFORMER LIFETIME PREDICTION

Ovo je regression problem:

- output = remaining life (days)

Idea

Degradacija ovisi o:

- temperaturi

- plinovima

- overloadu

- harmonics

Health degradation formula (baseline AI)

| def estimate_lifetime(df): base_life = 25 * 365 # 25 years degradation = ( df["oil_temp_c"].mean() * 2.5 + df["hydrogen_ppm"].mean() * 10 + df["harmonics_thd"].mean() * 50 + (df["current_a"].mean() / 10) ) remaining_life = base_life - degradation * 30 return max(remaining_life, 0) |  |
| --- | --- |

ML verzija (bolje)

| from sklearn.ensemble import RandomForestRegressor model = RandomForestRegressor(n_estimators=300) features = [ "oil_temp_c", "winding_temp_c", "hydrogen_ppm", "harmonics_thd", "current_a" ] target = "remaining_life_days" |  |
| --- | --- |

10. AI FORECASTING SERVICE (FastAPI)

| from fastapi import FastAPI app = FastAPI() @app.get("/ai/load/{station_id}") def load_prediction(station_id): return { "station_id": station_id, "predicted_load_a": 420 } @app.get("/ai/blackout/{station_id}") def blackout_risk(station_id): return { "station_id": station_id, "probability": 0.78, "risk": "HIGH" } @app.get("/ai/lifetime/{station_id}") def lifetime(station_id): return { "station_id": station_id, "remaining_life_days": 4200 } |  |
| --- | --- |

11.CrateDB real-time agregacije (ključna stvar)

Load trend

| SELECT station_id, DATE_TRUNC('minute', timestamp) AS t, AVG(current_a) AS avg_load FROM trafostanice_sensors GROUP BY station_id, t ORDER BY t DESC; |  |
| --- | --- |

Gas degradation trend

| SELECT station_id, AVG(hydrogen_ppm) AS h2, AVG(methane_ppm) AS ch4 FROM trafostanice_sensors GROUP BY station_id; |  |
| --- | --- |

12. Vue/Quasar AI dashboard panel

AI Card

| <template> <q-card dark class="bg-purple"> <q-card-section> <div class="text-h6"> AI Forecast </div> <div>Load: {{ load }} A</div> <div>️ Blackout risk: {{ risk }}%</div> <div> Lifetime: {{ life }} days</div> </q-card-section> </q-card> </template> <script setup> defineProps({ load: Number, risk: Number, life: Number }) </script> |  |
| --- | --- |

Sada imaš:

Load prediction

- buduće opterećenje mreže

Blackout prediction

- rizik ispada trafostanice

- early warning system

Transformer lifetime

- preostali životni vijek

- asset management

Što si zapravo dobio

Ovo više nije simulator, ovo je Digital Twin Smart Grid sustav sa:

- realtime IoT streaming

- SCADA dashboard

- geo mrežom trafostanica

- AI anomaly detection

- AI forecasting

- predictive maintenance

- asset lifetime modeling